TEMPERATURE CONTOLLER SCREEN

←    	ɷ    		↑    	↓

Set    	Advance	Up	Down

Press/ Hold ←       		for 3 seconds and release

CnPt / J1

Press ɷ 			tpun / F (Fahrenheit or Celsius)

Press ɷ 			tp-H / 165

Press ɷ 			tp-L / 50

Press ɷ 			CtrL / onoF

Press ɷ 			S-HC / Heat

Press ɷ 			ALA1 / 12

Press ɷ 			ALA2 / 0

Press ɷ 			C-SL / ASC11

Press ɷ 			CoSH / oFF

Press ɷ 			C-no / 1

Press ɷ 			bps / 9600

Press ɷ 			Len / 7

Press ɷ 			PrtY / EvEn

Press ɷ 			Stop / 1

Press ←

r-5 / ruN

Press ɷ 			AL1H / 0

Press ɷ 			AL1L / 0

Press ɷ 			Loc / off *

Press ɷ			Displays Temp/Set Point

*Change off to 2 and press ←

CHANGING HYSTERISYS (Temperature bandwidth)

Press ←   			HGS/ 5

Press ɷ			tp.of/ 0.0  (Adjust to change calibration offset)

CHANGING HIGH LIMIT TEMPERATURE SETTING

Press/ Hold ←    ɷ simultaneously for 3 seconds

Screen blinks once

Press/ Hold ← for 3 seconds

CnPt / J1 appears

Press ɷ

Press ɷ

Press ↑ to desired high limit set point

Press ←

Press ←

SETTING LOCK MODE

Press ɷ (4 times) until Loc / Off

All parameter changes are accomplished with the UP and DOWN arrow keys.

All Changes must be set into memory with the SET key (←)