Programming PM31

- Unlock Temperature Controller

- Go to Factory page

  - Press and hold ADVANCE key (green key with two circular arrows) and infinity (∞) key for six seconds

  - CUSt FCty is the first menu choice

  - Press ↑until you get to LoC FCty

- Unlocking Temperature Controller

  - Press ADVANCE  – LoC.o will appear

  - Change value to 0 using ↑↓

  - When in the LoC FCty screen, DO NOT change the oFF PAS.E setting as this will lock the controller with a random password

  - Press ADVANCE – rLoc5 will appear. Press ↑↓ to get value to 5

  - Press ADVANCE – SloC5 will appear. Press ↑↓ to get value to 5

  - Press ∞

  - Press up arrow until you get to CUSt FCty

  - Press ADVANCE and you will see sub-menu 1CUSt

  - Press ↑until you get to 3CUSt

  - Press ↑↓ to change PAr value to nonE

  - Press ∞ to go back to 3CUSt

  - Repeat these steps for 3CUSt through 10CUSt (all must have value nonE)

  - Press ∞ to return to standard temperature menu

- Hold ↑↓simultaneously until Ai oPEr appears

  - Press ↑↓ until LooP oPEr appears

  - Press ADVANCE until h.hY (hysteresis) appears. Value should be changed to 5

  - Press ∞ to get out of LooP oPEr mode

- Changing High Temperature Limit

  - Press ↑↓ simultaneously until Ai Set appears

  - Press ↓ until LooP Set appears

  - Press Advance key until 130 hsp appears

  - Press ↑ until value changes to 160

  - Press ∞ twice to return to main screen

- Locking Temperature Controller